Figure 5a. Cross-sectional view Alaskan Sugar Kelp Longline (grower fill in depth G and L)

H                                                                        (not to scale)

400 feet

E                                  F

I	7 feet

-	              G_____feet

D

C

J

B  _______feet                                       	K_____feet

A

A.  Mooring (200 lb. mushroom anchor or auger or granite block or  2000 lb. cement block)

B.  Grower fill in anchor line length and materials  (3/1 scope typical)

C.  7/16 inch seeded kelp poly line 400 feet long and 7 feet below the surface

D.  5-10 lb. cement weights or three holed bricks to keep kelp 7 feet below surface

E.  Surface mooring ball 18 inch diameter 100 lb. displacement

F.  5/16  inch poly depth 7’ control line (dropper) , 6x14 inch foam surface buoy and weight (D)

G. Water depth at low tide H.  Longline section (400 feet typical, grower fill in if different)

I. 7 feet ½ inch chain to shackle. J. Line holdfast K. Distance from kelp longline to bottom

Figure 5b. Detailed Drawing - Plan view Alaskan Sugar Kelp Longline

(not to scale)

D

A                             B                                                 C

A         Mooring

B         Mooring buoy

C         Kelp longline with surface buoys

D         400 foot kelp longline section (grower fill in if different)

Figure 5c. Detailed Drawing - Details on kelp depth control line dropper

A

D

B

C

A.  6 x 14 inch lobster buoy

B.  1 inch pvc pipe with a lobster spindle washer and figure 8 knot of 5/16 poly rope on each end

C.  10 lb. cement weight from ½ gallon paint bucket with knotted 5/16 poly loop or 3-holed brick

D.  7 foot length

Figure 5d.  Detailed Drawing - Anchoring system with configuration and poundage (fill in and /or add anchor drawing).

300  ft length

Buoy details: ______________

Anchor line details: _________________

Chain or cable details: __________________

Anchor details: ____________

Mooring detail to be filled out by grower in Figure 5.